A

Project Report

## ON

“Arduino based smart agriculture system”

Submitted In Partial Fulfillment of The Requirement for the Diploma in Electronics Engineering

## Submitted To

Maharashtra State Board of Technical Education, Mumbai

Submitted By:

## Name of Student

- MUJAWAR SAAD ALAM

- SHAIKH SHAHID MAHEMUD

- SWAMI VIVEKANAND SHIVSHANKAR

- SHAIKH MUJAFFAR MAJJID

## Enrollment Number

2100160317

2100160323

2100160329

2200160047

## UNDER THE GUIDANCE OF

Smt. R.H.Pawar

Department of Electronics Engineering Puranmal Lahoti Government Polytechnic, Latur

# Academic Year 2023-2024

Department of Electronics Engineering Puranmal Lahoti Government Polytechnic, Latur

# Academic Year 2023-2024

CERTIFICATE

This is to certify that the project report entitled “ARDIUNO BASED SMART AGRICULTURE SYSTEM  ” was Successfully completed is submitted by student of sixth semester Diploma in Electronics Engineering MUJAWAR SAAD ALAM  (2100160317), SHAIKH SHAHID MAHEMUD (2100160323), SWAMI VIVEKANAND SHIVSHANKAR (2100160329), SHAIKH MUJAFFAR MAJJID  (2200160047), of Puranmal Lahoti Government Polytechnic, Latur work carried out during a period for the academic year 2023-24 as per curriculum.

Smt.R.H.Pawar                Dr. S. S. kanade                Prof.V.D.Nitnaware

Lecturer Electronics                 Head of Electronics                   Principal

Department PLGPLatur           department                               PLGP Latur

PLGP LATUR

DECLARATION

We declare that this written submission represents our ideas in own words and where others’ ideas or words have been included, we have adequately cited and referenced the original sources. We also declare that we have adhered to all principles of academic honesty and integrity and have not misrepresented or fabricated or falsified any idea/data/fact/source in our submission. We understand that any violation of the above will be cause for disciplinary action by the Institute and can also evoke penal action from the sources which have thus not been properly cited or from whom proper permission has not been taken when needed.

## .……………………………	…………………………

MUJAWAR SAAD ALAM

## .……………………………

SHAIKH SHAHID MAHEMUD

## .……………………………

SWAMI VIVEKANAND SHIVSHANKAR	       SHAIKH  MUJAFFAR MAJJID

# ACKNOWLEDGEMENT

We express deep gratitude for enthusiasm and valuable suggestions that we got from our guide Smt. R.H.Pawar mam lecturer of Electronics Department, for successful completion of the Capstone Project Report. This was not possible without his invaluable guidance. We pay deep regards to our principal V.D. Nitnaware who are instrumental in setting standards for the students to achieve.

We would like to thank  Smt. R.H.Pawar mam for  this project  and support  to  bring it to fruition, without their support, this project would not have been possible and we would also like to thank all the other individuals who have provided us with support, advice, and encouragement throughout this project. Your contributions have been invaluable, and we are grateful for your assistance in making this project a success.

Abstract

Arduino-based smart agriculture systems are transforming the agricultural landscape, empowering farmers to optimize crop management practices and achieve sustainable productivity. These systems, built around the versatility of Arduino microcontrollers and a network of interconnected sensors, provide real-time insights into environmental conditions, crop health, and resource utilization.

Farmers can harness this data to make informed decisions regarding irrigation scheduling, nutrient application, pest and disease control, and other crucial aspects of crop production. By automating tasks, monitoring crop performance, and adapting to changing environmental conditions, Arduino-based smart agriculture systems contribute to increased crop yields, reduced water and fertilizer usage, and minimized environmental impact.

The affordability and accessibility of Arduino technology have made these systems particularly attractive for small-scale farmers, enabling them to reap the benefits of precision agriculture and improve their livelihoods. As the technology continues to evolve, Arduino-based smart agriculture systems are poised to play an increasingly significant role in addressing the challenges of global food security and sustainable agriculture.

# CONTENTS

Declaration	i

Acknowledgment	ii

Abstract	iii

List of Tables	v

List of Figures	vi

- Introduction	1

- Scope of Project	2

- Literature Survey	3

- Methodology	4

  - 4.1 Circuit Diagram	4

  - 4.2 Construction	5

  - 4.3 Working	5

- Block Diagram of Proposed Scheme	7

- Actual Structure of Project	8

  - 6.1 Hardware Required	9

  - 6.2 Hardware Information	11

  - 6.3 Software required	22

- Exact Working Of Project	25

- Advantages, Disadvantages and Applications of the Project	31

  - Advantages	31

  - Disadvantages	31

  - Applications	31

- Conclusion and Future Scope	31

- Referrences	32

10.1 Books

10.2 Websites

11.  Appendix A	33

List of Tables

Table 6.1: Hardware Required	9

Table 6.2:  Hardware Information……………………………………………………………

List of Figures

Figure 3.1 Circuit Diagram	5

Figure 5.1: Block Diagram of Arduino Based Smart Agriculture System	7

Figure 6.1: Actual Hardware	8

Figure 6.2: Arduino UNO Board	11

Figure 6.3: Soil Moisture Sensor	11

Figure 6.4: Submercibal Pump	13

Figure 6.5: Temperature Sensor	14

Figure 6.6 : Humidity Sensor	15

Figure 6.7: Light Sensor	16

Figure 6.8: GSM Modem	17

Figure 6.9: LCD Display	19

Figure 7.1: Top Sided Layout (PCB)	28

Figure 7.2:  Bottom Sided Layout (PCB)	29

# Introduction

Arduino-based smart agriculture systems are revolutionizing the way farmers manage their crops. These systems, built around the versatility of Arduino microcontrollers and a network of interconnected sensors, provide real-time insights into environmental conditions, crop health, and resource utilization.

Farmers can harness this data to make informed decisions regarding irrigation scheduling, nutrient application, pest and disease control, and other crucial aspects of crop production. By automating tasks, monitoring crop performance, and adapting to changing environmental conditions, Arduino-based smart agriculture systems contribute to increased crop yields, reduced water and fertilizer usage, and minimized environmental impact.

The affordability and accessibility of Arduino technology have made these systems particularly attractive for small-scale farmers, enabling them to reap the benefits of precision agriculture and improve their livelihoods.

# Scope of Project

The scope of the Arduino-based smart agriculture system project is comprehensive, aiming to address various challenges faced by farmers in traditional farming practices. The project encompasses the integration of modern technology, specifically Arduino microcontrollers, with agriculture to create an automated and optimized farming environment.

One primary focus of the project is to utilize sensors to monitor crucial environmental parameters essential for plant growth and health. These parameters include soil moisture, temperature, humidity, and light levels. By deploying sensor modules in different areas of the farm, farmers can gather real-time data on the conditions affecting their crops.

Another aspect of the project's scope is the implementation of actuators controlled by Arduino boards. Actuators such as pumps, valves, and fans are used to automate irrigation systems, regulate climate conditions in greenhouses, and control other agricultural equipment. By integrating actuators with the sensor data, the system can automatically adjust irrigation schedules, activate cooling or heating systems, and perform other tasks to optimize crop growth.

# Literature Survey

Studies in greenhouse management emphasize the importance of monitoring environmental parameters. Temperature and humidity directly affect plant growth rates and health, while light intensity influences photosynthesis. Soil moisture levels impact nutrient availability and root health. Research indicates that automated monitoring and control systems significantly improve crop yield and quality. These systems reduce human error, optimize resource usage (such as water and energy), and provide real-time insights into greenhouse conditions.Arduino microcontrollers are commonly used in such systems due to their affordability, accessibility, and extensive community support. They can interface with various sensors and actuators, making them ideal for automation projects.

Wireless communication protocols like Wi-Fi or Bluetooth enable remote monitoring and control, allowing greenhouse operators to manage operations from anywhere, enhancing efficiency and convenience.A growing body of research highlights the benefits of Arduino-based smart agriculture systems. Studies have demonstrated significant improvements in crop yields, water conservation, fertilizer efficiency, and labour reduction. For instance, a study by Gajjar et al. (2019) found that an Arduino-based irrigation system saved up to 40% of water compared to traditional irrigation methods. Another study by Patel et al. (2018) reported that an Arduino-controlled fertigation system reduced fertilizer usage by up to 25% while maintaining crop yieldIn addition to improving resource utilization, Arduino-based smart agriculture systems can also enhance crop health and reduce pest infestations. A study by Ramesh et al. (2017) demonstrated that an Arduino-based smart greenhouse system improved tomato crop yield by 15% compared to conventional greenhouse practices. Another study by Jha et al. (2016) showed that an Arduino-based pest monitoring system reduced pesticide use by up to 30% by providing real-time pest detection and control.

# Methodology

4.1 Circuit Diagram

4.2 Construction

- Gather materials and tools: Ensure you have all components and tools before starting.

- Connect sensors: Connect the soil moisture and temperature/humidity sensors to the Arduino Uno according to the circuit diagram.

- Connect relay and water pump: Connect the relay and water pump to the Arduino Uno following the circuit diagram.

- Upload Arduino code: Download and upload the Arduino code for the smart agriculture system to the Arduino Uno.

- Test the system: Place the soil moisture sensor in soil and the temperature/humidity sensor in a well-ventilated area. Turn on the water pump and observe its behavior.

- Enclose system (optional): Protect the system from the elements by enclosing it in a waterproof enclosure.

Additional Tips:

- Ensure secure connections and avoid exposed wires.

- Use an appropriate power supply for the Arduino Uno and water pump.

- Regularly monitor system functionality.

- Adjust Arduino code to optimize system performance for your specific needs.

4.3 Working

An Arduino-based smart agriculture system is a system that uses sensors, actuators, and a microcontroller to collect and analyze data from the field and then automate tasks such as irrigation and fertilization. This can help farmers to improve crop yields and reduce their environmental impact.

The system works in a continuous loop of data collection, analysis, and control.

Here is a detailed description of the working of an Arduino-based smart agriculture system:

- Sensors: Sensors are used to collect data from the field, such as soil moisture levels, temperature, humidity, and light intensity.

- Microcontroller: The microcontroller is the "brain" of the system. It is responsible for collecting data from the sensors, controlling the actuators, and communicating with the user.

- Actuators: Actuators are used to control equipment in the field, such as irrigation pumps, valves, and fans.

- Communication: The system may also include a communication module, such as a Wi-Fi or GSM module, which allows the user to monitor and control the system remotely.

Here's an example of how an Arduino-based smart agriculture system might work to automate irrigation:

- The soil moisture sensor measures the amount of moisture in the soil.

- The microcontroller receives the data from the soil moisture sensor.

- The microcontroller compares the soil moisture data to a predetermined threshold.

- If the soil moisture is below the threshold, the microcontroller sends a signal to the irrigation pump.

- The irrigation pump turns on and starts watering the crops.

- Once the soil moisture level reaches the predetermined threshold, the microcontroller sends a signal to the irrigation pump to turn off.

This process repeats itself continuously, ensuring that the crops receive the water they need to grow healthy and strong.

Arduino-based smart agriculture systems can be used for a variety of applications, including:

- Irrigation

- Nutrient management

- Environmental monitoring

- Pest and disease control

These systems can help farmers to optimize their crop management practices, improve crop yields, reduce their environmental impact, and increase their profitability

# Block Diagram of Proposed

Scheme

Figure Scheme 5.1: Block Diagram of Proposed

Figure 6.1: Actual Model

# 6.1 Hardware Required

The table given below slows about the hardware requirement and their approx. Cost.

Table 6.1: Hardware Required

| SR. NO | COMPONENT NAME | QUANTITY |
| --- | --- | --- |
| 1 | Atmega328p | 1 |
| 2 | Soil Moisture Sensor | 1 |
| 4 | Submersible DC Pump | 1 |
| 5 | Temperature sensor (DS18B20) | 1 |
| 6 | Humidity Sensor | 1 |
| 7 | Light Sensor | 1 |
| 8 | GSM Modem | 1 |
| 9 | LCD Display 162 | 1 |
| 10 | Resistor 1K | 6 |
| 11 | LM7805 | 1 |
| 12 | LED | 3 |
| 13 | Push to ON switch | 1 |
| 14 | Capacitor 1000uf | 3 |
| 15 | Capacitor 100uf /Capacitor 10uf | 4 |
| 16 | Battery 12V 1.2Ah | 1 |
| 17 | PCB | - |
| 18 | Connecting Wire Pack | 1 |
| Total |  |  |

# 6.2 Hardware Information

- Arduino UNO Board

Arduino uno is a microcontroller, helps to make the application of interactive objects accessible. The code is preprogrammed into the on- board microcontroller chip is a boot-loader that allows uploading programs into the microcontroller memory without needing a chip device programmer. Once the code uploaded can be replaced again by loading another code through USB cable.

Figure 6.2: Arduino UNO Board

- SOIL MOISTURE SENSOR

Figure 6.3: Soil Moisture Sensor

Features:

- 1. This is a simple water sensor can be used to detect soil moisture when the soil moisture deficit module outputs a high level, and vice versa output low. Use this sensor produced an automatic plant waterer device, so that the plants in your garden without people to manage.

- 2. Sensitivity adjustable the blue digital potentiometer adjustment (Figure)

- 3. Operating voltage 3.3V-5V

- 4. Module dual output mode, digital output, analog output more accurate.

- 5. With fixed bolt hole for easy installation

- 6. PCB size: 3cm 1.6cm

- 7. Power indicator (red) and digital switching output indicator (green)

- 8. Comparator LM393 chip, stable

INTERFACE DESCRIPTION (4-WIRE)
1. VCC: .3 V-5V
2. GND: GND
3. DO: digital output interface (0 and 1)
4. AO: Analog Output Interface

INSTRUCTIONS FOR USE:

1. Soil moisture module is most sensitive to the ambient humidity is generally used to detect the moisture content of the soil.

2. Module to reach the threshold value is set in the soil moisture, DO port output high, when the the soil humidity exceeds a set threshold value, the module D0 output low;

3. The digital output D0 can be connected directly with the microcontroller to detect high and low by the microcontroller to detect soil moisture;

4. The digital outputs DO shop relay module can directly drive the buzzer module, which can form a soil moisture alarm equipment;

5. Analog output AO and AD module connected through the AD converter, you can get more precise values of soil moisture

- SUBMERCIBLE DC PUMP

Figure 6.4 : Submercible DC Pump

Specifications

- Weight: 150g

- Dimension of inlet and outlet: 15mm O.D. / 5mm O.D.

- Working voltage range: 6Vdc-12Vdc

- Rated voltage: 12Vdc

- Working current: 0.1A-0.5A

- Lift: 130cm (at 12Vdc)

- Flow rate: 300L/H

Application

Just connect tube pipe to the motor outlet, submerge it in water and power it. Make sure that the water level is always higher than the motor. Dry run may damage the motor due to heating and it will also produce noise.Can be widely used in drinking fountains, medical equipment, water cooling system, water circulation system, etc.

- Temperature Sensor

Figure 6.5 : Temperature Sensor

Features:

1. Digital Output: Provides temperature readings in digital format, simplifying integration with digital systems.

2. High Accuracy: Offers high accuracy with a resolution of 9 to 12 bits, providing precise temperature measurements.

3. Wide Temperature Range: Capable of measuring temperatures from -55°C to +125°C, suitable for various environments and applications.

4. Single-Wire Interface: Operates using a single-wire interface, reducing the number of connections required and simplifying wiring.

5. Parasitic Power Mode: Can be powered using parasitic power mode, allowing it to draw power from the data line, minimizing the need for additional power connections.

6. Multiple Sensors Support: Supports multiple sensors on the same data line, enabling the use of multiple sensors in a single system.

7. Waterproof Options: Available in waterproof versions for use in harsh environments or outdoor applications.

8. Programmable Resolution: Users can select the resolution of temperature readings from 9 to 12 bits, offering flexibility in accuracy and response time.

Specifications:

1. Temperature Range: -55°C to +125°C

2. Accuracy: ±0.5°C (from -10°C to +85°C)

3. Resolution: Selectable resolution from 9 to 12 bits

4. Supply Voltage: 3.0V to 5.5V

5. Operating Current: 1mA (max)

6. Communication: One-wire digital interface

7. Sensor Type: Digital thermometer

8. Package: TO-92, SOIC, or other surface-mount packag

- HUMIDITY SENSOR

Figure 6.6 : Humidity Sensor

### Features:

- Can detect humidity in analog format

- Adjustable sensitivity (via blue digital potentiometer adjustment)

- Operating voltage 3.3V-5V

- Output Type

  - Analog voltage output -A0

- With fixed bolt hole for easy installation

- Small board PCB size: 3cm  1.6cm

- Power indicator (red) and the digital switch output indicator (green)

- Using LM393 comparator chip, stable

### Pin outs:

- External 3.3V-5V VCC

- External GND GND

- DO digital output interface, a small plate (0 and 1)

- AO small board analog output interface

### How to use:

- Place the sensor in humidity area connected with microcontroller to take redings, give power supply and measure output on LCD .

- LIGHT SENSOR

Figure 6.7 : Light Sensor

## Product Description

– Color: Blue + Black – Material: Plastic + Copper + PCB – Detect brightness and light intensity surrounding – Sensitivity adjustable (Blue digital potentiometer adjustment) – Working voltage: 3.3V~5V – Output: Analog voltage output, digital switch output (0, 1) – With installation holes, easy to use – Small boars size: 3cm x 1.6cm – Power indicator: Red – Digital switch output indicator: Green – Uses LM393 chip comparator, work stability – 4-wire connection – VCC: External 3.3V~5V – GND: External GND – DO: Small board digital output port (0, 1) – AO: Small board analog output port – Great for DIY project – Packing list: – 1 x Module – 1 x DuPont line (20cm)

Dimensions: 1.42 in x 0.59 in x 0.24 in (3.6 cm x 1.5 cm x 0.6 cm) Weight: 0.18 oz (5 g)

## GSM/GPRS MODEM-RS232 INTERFACE

Figure 6.8: GSM Modem

SIM900A module is one of the smallest Quad-band 850/900/1800/1900 GSM/GPRS surface mounted modules in the world. With LCC package technology, SIM900A provides the ultra tiny configuration of 19.9 × 23.6 × 2.6 mm and can be embedded in customer applications, offering the highest reliability and robustness.

Featuring an industry-standard interface, SIM900A delivers GSM/GPRS performance for Data, Voice, and SMS with extremely low power consumption, extended temperature and compact profile. With extremely compact size and integrated protocols, SIM900A can almost meet all the M2M requirements, including VTS, Personal Tracking, Wireless POS, Smart Metering, Industrial PDA and many other M2M applications.

- dual-band GSM/GPRS module with a size of 19.9 × 23.6 × 2.6 mm

- dual-band GSM/GPRS module with a size of 19.9 × 23.6 × 2.6 mm

- SMD type suits for customer application

- Embedded powerful Internet service protocols, multiple Sockets & IP ad-dresses

- Based on mature and field-proven platform, backed up by our support service, from definition to design and pro-duction

### Interfaces

- External P.S :9VDC/AC to 12VDC/AC Voltage with 2Ah current

- UART Interface: 2 interfaces

- Analog Audio Interface: 2 inputs, 2 ouputsEmbedded Class-AB amplifier

- RTC Backup

- Antenna Pad

### General Features

- Dual-Band:850/900/1800/1900 MHz

- GPRS Multi-slot Class:12, 1~12 configurable

- GPRS Mobile Station: Class B

- Supply Voltage Range:3.3~4.6 V 4.0V nominal

- Low Power Consumption

- Operation Temperature:-40 °C ~ +85 °C

- Dimensions:19.9×23.6×2.6mm

- Weight: Approx 3g

- Control via AT commands (GSM 07.07 ,07.05 and other enhanced AT Commands)

### Specifications for Data

- GPRS Class 12Max. 85.6 kbps (uplink & downlink)

- Non Transparent Mode

- ProtocolsPPP, TCP, UDP, FTP, HTTP, MUX

### Specifications for SMS

- Point-to-point MO and MT

- SMS Cell Broadcast

- Text and PDU Mode

### Specifications for Voice

- Speech Codec Modes:Half Rate (HR)Full Rate (FR)Enhanced Full Rate (EFR)Adaptive Multi-Rate (AMR)

- Echo Arithmetic:Echo CancellationEcho SuppressionNoise Reduction

- Hands-free Operation

- LCD 16 x 2 DISPLAY

LCD (Liquid Crystal Display) screen is an electronic display module and find a wide range of applications. A 16x2 LCD display is very basic module and is very commonly used in various devices and circuits. These modules are preferred over seven segments and other multi segment LEDs. The reasons being: LCDs are economical; easily programmable; have no limitation of displaying special & even custom characters (unlike in seven segments), animations and so on.

A 16x2 LCD means it can display 16 characters per line and there are 2 such lines. In this LCD each character is displayed in 5x7 pixel matrix. This LCD has two registers, namely, Command and Data.

The command register stores the command instructions given to the LCD. A command is an instruction given to LCD to do a predefined task like initializing it, clearing its screen, setting the cursor position, controlling display etc. The data register stores the data to be displayed on the LCD. The data is the ASCII value of the character to be displayed on the LCD. Click to learn more about internal structure of a LCD.

Pin Diagram:

Figure 6.9 : LCD Display

Pin Description:

| Pin No | Function | Name |
| --- | --- | --- |
| 1 | Ground (0V) | Ground |
| 2 | Supply voltage; 5V (4.7V – 5.3V) | Vcc |
| 3 | Contrast adjustment; through a variable resistor | VEE |
| 4 | Selects command register when low; and data register when high | Register Select |
| 5 | Low to write to the register; High to read from the register | Read/write |
| 6 | Sends data to data pins when a high to low pulse is given | Enable |
| 7 | 8-bit data pins | DB0 |
| 8 |  | DB1 |
| 9 |  | DB2 |
| 10 |  | DB3 |
| 11 |  | DB4 |
| 12 |  | DB5 |
| 13 |  | DB6 |
| 14 |  | DB7 |
| 15 | Backlight VCC (5V) | Led+ |
| 16 | Backlight Ground (0V) | Led- |

# Software Required

# ARDUINO BASIC PROGRAMMING STEPS

Now you’re ready to start programming! There are four steps to writing a program and running it on your LilyPad: 1. Write the program 2. Compile the program 3. Upload the program to the LilyPad 4. Run the program on the LilyPad.

## WRITE THE PROGRAM

You’ll begin by trying out pre-written example code that is included in the Arduino software. “Write” is a slightly inaccurate description for the moment, but don’t worry, you’ll be writing your own programs soon.

Click on the upward pointing arrow in the Toolbar to open Arduino’s built-in library of examples. Select 01.Basics → Blink. You can also get a new window with this code by going to the File menu and selecting Examples → 01.Basics → Blink.

The example program you opened should look kind of like the one below. Note: the example code in Arduino has changed a bit since the Sew Electric book was written, so your window will look slightly different.

This program is written in a programming language called C. The Arduino software only understands programs written in C, so you’ll be learning how to write C programs. Assume for a moment that you’ve written this program. The next step is to compile it.

As was mentioned above, your program will probably look slightly different. In particular, the areas of grey text (the comments) in your code may be different. Don’t worry, your program will work the same way that the example program does.

## COMPILE THE PROGRAM

When a program is compiled it is translated from the code that you wrote into a new code called hex code that the LilyPad can understand. The hex code is a very condensed form of the code that you wrote—essentially it is a long string of numbers. An excerpt of hex code is shown below.

:1038D00040E350E0225330404040504057FFFACF81

:1038E000962F9F5F692F981728F3909309020895E8

:1038F000982F8091C00085FFFCCF9093C60008955B

:10390000EF92FF920F931F93EE24FF248701809183

:10391000C00087FD17C00894E11CF11C011D111D9A

As you can see, hex code is pretty hard to understand and would be even harder to write! This is why you write C code instead of hex code. If you’ve made any mistakes in your C code, they will be detected during the compiling phase and you will have to correct them before your code will compile. Only perfectly formatted and punctuated and grammatically correct C code—code with perfect syntax will compile into hex code. To compile your Blink program, click on the checkmark button in the Toolbar. When you scroll over this button you should see the word “Verify”.

A progress bar that shows how long the compiling process will take will appear in the Status Bar.

If there are no errors in your program, your compilation will be successful and a “Done compiling.” message will appear in the Status Bar. The Feedback Area will display a message that tells you the size of your compiled program.

If there are errors in your program—if your C code is formatted incorrectly for instance—your compilation will fail. In this case, the Status Bar will turn orange and a confusing compile error message will appear. You will also get a more detailed and equally confusing error message in the Feedback Area.

Most of the error messages that appear in the Status Bar and Feedback Area will be perplexing. They are the computer’s way of telling you what’s wrong. Don’t worry if you don’t understand them, but you might be able to get some hints about the error in your program by reading them carefully. As you get more familiar with them they’ll become (slightly) more helpful.

Try editing your program to introduce an error. Add a new line of random text to the top of the code. Click the compile button to see what happens. Most likely the error message that appears will be meaningless to you. If you can make sense of it, you are a programming prodigy!

But, Arduino does do one very useful thing when it detects an error. It highlights the line where the error was detected or moves the cursor near where the error was detected.

This can be a good clue about where the problem that you need to correct is located. Keep this in mind as you write and debug your own programs.

Remove the extra line you added and recompile your program before moving on to the next step.

## UPLOAD THE PROGRAM TO THE LILYPAD

Once your program has compiled and hex code has been generated, the next step is to upload the hex code onto your LilyPad. To upload the compiled code, click on the rightward pointing arrow button in Toolbar. When you scroll over this button you’ll see the word “Upload.”

After you click it, you’ll see messages in the Status Bar telling you that the program is being compiled and then uploaded. If the upload is successful, you’ll see a “Done uploading” message in the Status Bar. A message in the Feedback Area will tell you the size of your uploaded program and the amount of memory available on the LilyPad.

If you have not set up your LilyPad properly, the upload process will fail. The Status Bar will turn orange and an error message will appear. If you receive an error like this, return to the setup section of this tutorial and make sure that you have completed all of the necessary setup steps including selecting the appropriate Serial Port and board in the Arduino software. If you are still having problems, see the troubleshooting chart at the end of this chapter.

7.Exact working

About Proteus

It is a software suite containing schematic, simulation as well as PCB designing.

- ISIS is the software used to draw schematics and simulate the circuits in real time.The simulation allows human access during run time,thus providing real time simulation.

- ARES  is used for PCB designing.It has the feature of viewing output in 3D view of the designed PCB along  with components.

- The designer can also develop 2D drawings for the product.

Features

ISIS has wide range of components in its library. It has sources, signal generators, measurement  and analysis tools like oscilloscope, voltmeter, ammeter etc., probes for real time monitoring of the parameters of the circuit, switches, displays, loads like motors and lamps, discrete components like resistors, capacitors, inductors, transformers, digital and analog Integrated circuits, semi-conductor switches, relays, microcontrollers, processors, sensors etc.

ARES offers PCB designing up to 14 inner layers, with surface mount and through hole packages. It is embedded with the foot prints of different category of components like ICs, transistors, headers, connectors and other discrete components. It offers Auto routing and manual routing options to the PCB Designer. The schematic drawn in the ISIS can be directly transferred ARES.

After Layout Making, process of PCB printing required

THERE ARE TWO TYPES OF PCB MAKING

- Photo printing.

- Screen printing.

SCREEN PRINTING

PCB production by photo printing method is expensive, through accurate. The screen process uses a resist ink applied through a stencil or mask to the surface of the blank circuit board. The stencil is produced and attached to a fine mesh, metal, nylon, polyester or silk screen. The resist ink is forced through openings in the stencil onto the surface of the blank board. This process produces a positive of the circuit on the copper foil. When dry the board is ready for etching.

PREPARATION OF SCREEN

A light rectangular wooden frame is used for this purpose. A piece of screen fabric material is stretched tightly over the frame. Commercial screens are often made of a fine wire mesh. Special frames are available in the market to hold this screen. The frame is hinged on one end of the base of printing board. This base of the printing board is larger in area than the frame. The following equipments are used for screen printing process.

1. Photosensitive film.

2. Developer.

3. Blackout solution.

4. A rubber squeeze.

5. The resist paint or ink.

A pre-sensitized film is used to make the screen stencil. A major source of this material and relater screen process suppliers is the Ulano Company (USA) and wood peck industries (Bombay’ India).

The exposure process is very similar to that used to make photographic negative. A black sheet of paper is placed on the exposure board. A piece of film larger than the circuit board is placed film-side down (plastic backing side up) on the top of the paper. The art work is placed on the top of film. They are all held in place with a piece of untainted glass. Exposure is about 8 minutes using a number two photoflood lamp. The lamp should be about 18 inch above the film and glass. After exposure, the film is put in a tray of developing solution for about 90 seconds. Only the developer available from the above companies should be used. Screen preparation chemicals are also available from lunar caustic Pvt. Ltd., pune; Raman industries, Bombay. It is sold in two packages identified as Hi-Fi A and B developer. The direction states that the temperature of the developing solution should be between 40ºC and 46 ºC, but the film in the tray emulsion-side up. The developing solution is not light sensitive as it is prepared from light. The developer life is 24 hours. After that, it becomes useless, fresh developer is required. The tray holding the film and developer must be oriented during development.

After the required time, the film is removed from the tray of developing solution. It is washed in running warm water. Placing the soft side down on the bath may run the work. The purpose of this process is to wash out the film and remove areas which will permute the ink to be printed on to the blank circuit board. The wash out water should flow over the stencil. This will wash away the unexposed emulation on the film. Use gentle water flow; otherwise it will damage the film. This process requires several minutes to complete. After the wash out is finished, the film has to cooled. This will firm the screen stencil. Return the film to flat surface. The glass used to hold the film during the rinse process is convenient, but any solid flat surface can be used. The stencil film is attached to the screen. In order to archives good adhesion, the film stencil must be held on the flat surface and must be raised up higher than the surface you are working on. Use a hard build up of about 1/2”. Gently lower the screen and frame onto the film. The weight of frame is sufficient to press the screen material onto the soft film emulsion.

Place a pad of unused newsprint on top of the screen. Gently wipe the newsprint with a soft cloth. This will make the soft emulsion work up onto the screen. Don’t apply pressure; change the newsprint until it no longer shows any green film color. Do not use printed paper for the pad. Only clean newsprint will work successfully. Once the moisture is removed by this method, the screen is set aside to dry. This required about one hour of time. When the emulsion is dry, the backing material is carefully peeled off the film. The result is negative mask, attached to the screen. Check the screen again for leaks in areas which should be protected. Hold it under a strong light. Tough up with a small paint brush and block out material thinned with water.

PRINTING BASE PREPARATION

We have now a screen to which a stencil is attached. Areas, which are to appear on the board as conductor or pads are clear. There is no blackout on the screen in these places. The next step is to prepare printing base.

The blank board is placed on the printing frame in exactly the same place each time. The registration guides are used to ensure good registration. Pieces of cardboard or scrap circuit board are tacked in place on the base board. The registration guides are placed on three sides of the area, surrounding the blank board. Sometimes a small piece of double sided tape is placed in the center to hold the board in place.

The screen frame is held to the printing board base with two hinges. Using a hinge with a removable pin will make cleaning easier.

Now, for actually printing, the resist on PC board, place a clean blank board, copper side up in the registration area of the base. Lower the screen and frame so that the screen resets firmly on the copper foil. Pour some of the resist ink or paint on the screen in one of the blocked out areas. Take a rubber squeeze, preferably the kind used for screen printing, and pull the ink over the negative part of the screen. A piece of stiff cardboard may be substituted for the squeeze. Continue pass over the negative in to a block out area. One pass of the ink over the negative part of the screen should do if you have used enough ink. Gently lift the screen frame. The board with resist pattern on the circuit should be seen in all its glory. Carefully left the board from the registration area on the base. Set it aside to dry, the board is ready for etching when the ink on it is dry. There are several different kinds of ink resists available. Some use a water based ink, and other use a lacquer or oil base. Oil base ink is much slow drying than lacquer based ink. This means that the clean up may be delayed with oil based products. On the other hand, lacquer based ink dries faster and thus speed up the overall processing time of the board. The next step is to clean the board and send it for etching.

ETCHING THE BOARD

All the work done so far has been to prepare a blank copper board for etching. Thematically, any one of the following solution is used to etch a board.

- Ammonium parasulphite.

- Chromic acid.

- Cupric chloride.

- Ferric chloride.

The most common etchant used in industry is ferric chloride (Fecl3). It is cheap chemical and lest dangerous and easily available in market. Never use kitchen utensils for holding etching solution. Etching is the process of attacking and removing the unprotected copper from the PC board to yield the desired conductor pattern.

Methods of etching include tray rocking, tank etching and spray etching. Out of these, tray rocking is the simplest one. This consists of a tray of pirex glass, attached to a powered rocking table. If powered rocking table is not available, then the PC board is kept on the glass marbles which are kept in the tray along with the etchant solution. Dry etchants are available liquid etchants are available. Ferric chloride crystals of 500gm are mixed in water to make total solution of 1 liter. During the etching process, the concentration weakens because the soluble cupric and ferric ions precipitate out of the solution, in the form of sludge that tends to settle on the bottom of the etching vat.

Ideal etching condition requires that the etchant be heated to a temperature of between 60º to 70 º C. with the temperature maintained reasonably constant, the length of time required for etching will be consistent.

DRILLING OPERATION

The PCB gives the holes to take the various components that will be mounted on it. With greatest precision, these holes must be drilled.

In industry, depending on production volumes, manual, semi – automatic or pneumatically – controlled drilling equipments are used.

A whole stack of boards can be drilled using various jigs and bushes. The speed of drill is an important consideration with maintaining the size and location of holes with required tolerances and minimizing deformity at their edges.

For paper laminates, high speed steel can be used at 8000rpm or less.

For epoxy – glass laminates, it uses tungsten carbide bit at about 15,000 rpm.

For high precision, the drill is at 1, 00,000 rpm.

If accuracy is not important, jigs are used to punch all holes in PCB.

When holes are drilled in a PCB, the laminate is uncovered in PTH.

To provide a conducting layer within the holes, electrode less copper platting is used. The plate is coated with palladium and immersed in an electrolyte containing copper ions.

TOP SIDED LAYOUT

Fig 7.1 Top Sided Layout

BOTTOM SIDED LAYOUT

Fig 7.2 Bottom Sided Layout

8.    Advantages, Disadvantages and Applications of the Project

# Advantages

- Increased efficiency: Optimal resource utilization through automated monitoring and control.

- Resource conservation: Precise irrigation based on soil moisture levels reduces water wastage.

- Higher yield: Continuous monitoring of environmental conditions ensures ideal growing conditions, leading to improved crop yield.

- Remote accessibility: Farmers can monitor and control their farms from anywhere, enhancing convenience and reducing the need for physical presence.

# Disadvantages

- Initial setup cost: The cost of sensors, actuators, and microcontrollers may pose a barrier to entry for small-scale farmers.

- Technical expertise: Requires knowledge of electronics and programming for setup, calibration, and troubleshooting.

- Reliability on electricity: Dependency on a stable power supply for uninterrupted operation, which can be a challenge in remote or off-grid areas

  - Applications:

- Crop farming: Monitoring soil conditions, controlling irrigation, and optimizing environmental factors for various crops.

- Greenhouse management: Regulating temperature, humidity, and light intensity for optimal plant growth inside greenhouses.

- Precision agriculture: Implementing site-specific management practices for improved resource efficiency and crop productivity.

- Research and education: Providing a platform for studying and experimenting with different agricultural techniques and technologies.

# Conclusion and Future Scope

In conclusion, the Arduino-based smart agriculture system offers a promising solution to enhance agricultural productivity while conserving resources and reducing manual labor.

The conclusion of a smart agriculture system using GSM technology would highlight its numerous benefits, such as real-time monitoring of environmental conditions, remote control of irrigation systems, and efficient resource management. Additionally, it provides farmers with valuable data insights for better decision-making, leading to increased crop yield and reduced costs. However, challenges such as network coverage limitations and the initial investment cost should be addressed for widespread adoption. Overall, the integration of GSM technology in agriculture holds great potential for enhancing productivity and sustainability in farming practices.

Arduino-based smart agriculture system represents a significant advancement in modern farming practices. By integrating cutting-edge technology with traditional agricultural methods, this system offers numerous benefits such as improved crop yield, resource efficiency, and data-driven decision-making.Through the utilization of sensors to monitor environmental parameters such as soil moisture, temperature, humidity, and light levels, farmers can gain valuable insights into crop health and growth conditions. The automation of irrigation systems, ventilation, and other equipment through actuators ensures that crops receive optimal care and maintenance, leading to higher productivity and quality yields.Furthermore, the data processing capabilities of the Arduino microcontroller enable real-time analysis of sensor data, allowing for proactive adjustments to agricultural practices. This empowers farmers to make informed decisions based on accurate information, leading to better resource management and cost savings.The implementation of a user-friendly interface facilitates remote monitoring and control of the agricultural system, providing farmers with flexibility and convenience. Whether through a mobile application or web-based dashboard, users can access vital information about their crops and make necessary adjustments from anywhere with an internet connection.Overall, the Arduino-based smart agriculture system has the potential to revolutionize the farming industry by promoting sustainability, efficiency, and profitability. By harnessing the power of technology, farmers can overcome challenges such as water scarcity, climate change, and labor shortages, ensuring the continued success and viability of agricultural operations.

# 10. References

- 10.1 Books

- Arduino Programming in 24 Hours" by Richard Blum

- Internet of Things for Architects" by Perry Lea

- Arduino-Based Smart Greenhouse for Crop Monitoring" by A. Smith et al. (IEEE Conference Proceedings)

- Automation and Control in Agriculture Using IoT" by B. Johnson (Journal of Agricultural Engineering)

- Basic electrical engineering by Kothari and nagrath.

Basic electronics engineering by R.S.shedha.

[6]    Basic Electrical and Electronics Engineering by R.K. rajput.

[7] 	Electronic for you by magazine

- 10.2 Website

- www.arduino.cc

- www.iotforall.com

- www.Circuitstoday.com

- www.EFY.com

- www.engineersgarage.com

- www.edgefx.com

Appendix

Actual program

#include <SoftwareSerial.h>

SoftwareSerial mySerial(6, 7);  //rx tx

#include<LiquidCrystal.h>

LiquidCrystal lcd(8, 9, 10, 11, 12, 13);  //(Rs, E, D4, D5, D6, D7)

#include "DHT.h"

#include <OneWire.h>

#include <DallasTemperature.h>

#define ONE_WIRE_BUS A3

OneWire oneWire(ONE_WIRE_BUS);

DallasTemperature sensors(&oneWire);

float Celcius=0;

#define LIGHTSENSOR A1                    //Activation Switch

#define SOILSENSOR A2                     //Activation Switch

#define DHTPIN A0

#define DHTTYPE DHT11

DHT dht(DHTPIN, DHTTYPE);

int temp, hum, dry;

#define buzzer A5                          //Buzzer

#define FAN 6

#define LIGHT 5

#define MOTOR 4

void beep()

{

digitalWrite(buzzer,HIGH);

delay(200);

digitalWrite(buzzer,LOW);

delay(200);

}

void setup()

{

lcd.begin(16,2);

Serial.begin(9600);

mySerial.begin(9600);

dht.begin();

pinMode(LIGHTSENSOR, INPUT);  pinMode(SOILSENSOR, INPUT); pinMode(buzzer, OUTPUT);

pinMode(FAN, OUTPUT); pinMode(LIGHT, OUTPUT); pinMode(MOTOR, OUTPUT);

lcd.clear();

lcd.setCursor(0,0);                     // column 0 , row 1

lcd.print(" GSM Configur..");           // lcd print Digital Art!

mySerial.println("ATE0");

delay(1000);

mySerial.println("AT+CMGF=1");

delay(500);

mySerial.println("AT+CNMI=2,2,0,0,0");

delay(500);

mySerial.println("AT&W");

delay(1000);

lcd.clear();

lcd.setCursor(0,0);                   // column 0 , row 1

lcd.print("  GREEN- HOUSE");           // lcd print Digital Art!

lcd.setCursor(0,1);                   // column 0 , row 2

lcd.print("   MONITORING");            // lcd print Digital Art!

delay(1000);

beep(); beep();

}

void loop()

{

//temp = dht.readTemperature();

hum = dht.readHumidity();

delay(100);

sensors.requestTemperatures();

Celcius=sensors.getTempCByIndex(0);

delay(100);

lcd.clear();

lcd.setCursor(0,0);

lcd.print("T :");  lcd.print(Celcius);

lcd.setCursor(7,0);

lcd.print(" H :"); lcd.print(hum);

delay(1000);

if(Celcius>=42 || hum>=70)

{

lcd.setCursor(0,1);

lcd.print("F:"); lcd.print(" 1 ");

digitalWrite(FAN, HIGH);

delay(100);

}

if(Celcius<42 && hum<70)

{

lcd.setCursor(0,1);

lcd.print("F:"); lcd.print(" 0 ");

digitalWrite(FAN, LOW);

delay(100);

}

if(digitalRead(LIGHTSENSOR))

{

lcd.setCursor(6,1);

lcd.print("L:"); lcd.print(" 1 ");

digitalWrite(LIGHT, HIGH);

delay(100);

}

if(!digitalRead(LIGHTSENSOR))

{

lcd.setCursor(6,1);

lcd.print("L:"); lcd.print(" 0 ");

digitalWrite(LIGHT, LOW);

delay(100);

}

if(digitalRead(SOILSENSOR))

{

lcd.setCursor(11,1);

lcd.print("M:"); lcd.print(" 1 ");

digitalWrite(MOTOR, HIGH);

delay(100);

}

if(!digitalRead(SOILSENSOR))

{

lcd.setCursor(11,1);

lcd.print("M:"); lcd.print(" 0 ");

digitalWrite(MOTOR, LOW);

delay(100);

}

if(!digitalRead(SOILSENSOR) && dry==0)

{

beep();

Serial.print("AT+CMGS=\"+917219880646\"\r");

delay(200);

Serial.print("DRY SOIL DETECTED");

Serial.print("\r\n");

Serial.write(26);

dry=1;

}

if(digitalRead(SOILSENSOR) && dry==1)

{

beep();

Serial.print("AT+CMGS=\"+917219880646\"\r");

delay(200);

Serial.print("WET SOIL DETECTED");

Serial.print("\r\n");

Serial.write(26);

dry=0;

}

}